HW#8

Mrinmoy Sarkar

ECEN-621

Q1: Code:

#include "msp.h" #define MAX_ADC_VALUE 16384 int period_of_blue_led = 16384; // fixed period for blue led volatile uint16_t JOYresults[2]; // store the ADC value of the joystick float duty_cycle = 0.5; // store the instantaneous duty cycle of the two led int main(void) { WDT_A->CTL = WDT_A_CTL_PW ¦ WDT_A_CTL_HOLD; // Stop WDT // Configure GPIO for PWM output P2->DIR ¦= BIT6 ¦ BIT7; // red led and buzzer P2->SEL0 ¦= BIT6 ¦ BIT7; P5->DIR ¦= BIT6; // blue led P5->SEL0 ¦= BIT6; // configure timer for pwm operation TIMER_A0->CCR[0] = 1000 - 1; // PWM Period TIMER_A0->CCTL[3] = TIMER_A_CCTLN_OUTMOD_7; // CCR3 reset/set TIMER_A0->CCR[3] = 750; // CCR3 PWM duty cycle TIMER_A0->CCTL[4] = TIMER_A_CCTLN_OUTMOD_7; // CCR4 reset/set TIMER_A0->CCR[4] = 500; // CCR4 PWM duty cycle TIMER_A0->CTL = TIMER_A_CTL_SSEL__SMCLK ¦ // SMCLK TIMER_A_CTL_MC__UP ¦ // Up mode TIMER_A_CTL_CLR; // Clear TAR TIMER_A2->CCR[0] = period_of_blue_led - 1; // PWM Period TIMER_A2->CCTL[1] = TIMER_A_CCTLN_OUTMOD_7; // CCR1 reset/set TIMER_A2->CCR[1] = 750; // CCR1 PWM duty cycle TIMER_A2->CTL = TIMER_A_CTL_SSEL__SMCLK ¦ // SMCLK TIMER_A_CTL_MC__UP ¦ // Up mode TIMER_A_CTL_CLR; // Clear TAR // Configure GPIO for ADC P4->SEL1 ¦= BIT4; // Enable A/D channel A9 P4->SEL0 ¦= BIT4; P6->SEL1 ¦= BIT0; // Enable A/D channel A15 P6->SEL0 ¦= BIT0; // Turn on ADC14, extend sampling time to avoid overflow of results ADC14->CTL0 = ADC14_CTL0_ON ¦ ADC14_CTL0_MSC ¦ ADC14_CTL0_SHT0__192 ¦ ADC14_CTL0_SHP ¦ ADC14_CTL0_CONSEQ_3; ADC14->MCTL[0] = ADC14_MCTLN_INCH_9; // ref+=AVcc, channel = A9 ADC14->MCTL[1] = ADC14_MCTLN_INCH_15 ¦ ADC14_MCTLN_EOS; // ref+=AVcc, channel = A15, end seq. ADC14->IER0 = ADC14_IER0_IE1; // ADC interrupt enable for ADC14->MEM[1] SCB->SCR ¦= SCB_SCR_SLEEPONEXIT_Msk; // sleep on exit // Start conversion-software trigger ADC14->CTL0 ¦= ADC14_CTL0_ENC ¦ ADC14_CTL0_SC; NVIC->ISER[0] = 1 << ((ADC14_IRQn) & 31); // Enable ADC interrupt in NVIC module // Enable global interrupt __enable_irq(); // Enter LPM0 __sleep(); } // ADC14 interrupt service routine void ADC14_IRQHandler(void) { if (ADC14->IFGR0 & ADC14_IFGR0_IFG1) { JOYresults[0] = ADC14->MEM[0]; // Move A9 results, IFG is cleared. vertical joystick JOYresults[1] = ADC14->MEM[1]; // Move A15 results, IFG is cleared. horizontal joystick duty_cycle = (float)JOYresults[1] / MAX_ADC_VALUE; // calculate the duty cycle TIMER_A0->CCR[0] = JOYresults[0] - 1; // reset the PWM Period for buzzer TIMER_A0->CCR[3] = duty_cycle * JOYresults[0]; // reset the duty cycle of red led TIMER_A0->CCR[4] = 0.5*JOYresults[0]; // set the buzzer duty cycle to 50% TIMER_A2->CCR[1] = duty_cycle * period_of_blue_led; // reset the duty cycle of blue led __delay_cycles(2000); // small delay } }

Q1: UML diagram: